CCNA: SRWE

Skills Assessment (OSPF) – Practice Exam

Topology

VLAN Key

| VLAN | Name | Network |
| --- | --- | --- |
| 31 | Sales | 172.10.31.0 /24 |
| 32 | Marketing | 172.10.32.0 /24 |
| 12 | Management | 172.10.12.0/24 |

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0.12 | 172.10.12.253 | 255.255.255.0 | N/A |
|  | G0/0/0.31 | 172.10.31.253 | 255.255.255.0 | N/A |
|  | G0/0/0.32 | 172.10.32.253 | 255.255.255.0 | N/A |
|  | G0/0/1 | 192.168.12.1 | 255.255.255.252 | N/A |
|  | Lo1 | 172.10.1.1 | 255.255.255.0 | N/A |
|  | Lo2 | 172.10.2.1 | 255.255.255.0 | N/A |
|  | Lo3 | 172.10.3.1 | 255.255.255.0 | N/A |
| S3 | G0/1 | 192.168.12.2 | 255.255.255.252 | N/A |
|  | G0/2 | 192.168.12.5 | 255.255.255.252 | N/A |
|  | Lo0 | 200.7.12.18 | 255.255.255.240 | N/A |
| R3 | G0/0.12 | 172.10.12.252 | 255.255.255.0 | N/A |
|  | G0/0.31 | 172.10.31.252 | 255.255.255.0 | N/A |
|  | G0/0.32 | 172.10.32.252 | 255.255.255.0 | N/A |
|  | G0/1 | 192.168.12.6 | 255.255.255.252 | N/A |
|  | Lo4 | 172.20.44.4 | 255.255.255.0 | N/A |
|  | Lo5 | 172.20.45.5 | 255.255.255.0 | N/A |
| S1 | VLAN 12 | 172.10.12.31 | 255.255.255.0 | 172.10.12.254 |
| S2 | VLAN 12 | 172.10.12.32 | 255.255.255.0 | 172.10.12.254 |
| PC-A | NIC | DHCP |  |  |
| PC-B | NIC | DHCP |  |  |

Assessment Objectives

Part 1: Configure Device Basic Settings

Part 2: Configure LAN Redundancy and Link Aggregation

Part 3: Configure HSRP

Part 4: Configure DHCP

Part 5: Configure OSPF Dynamic Routing Protocols

Part 6: Verify Network Connectivity

Scenario

In this Skills Assessment, you will create a small network. You must connect the network devices, and configure those devices to support IPv4 connectivity, LAN redundancy, DHCP addressing and link aggregation. You will then configure OSPF and verify connectivity.

Time – 90 minutes

Required Resources

3 Routers (Cisco 1941)

2 Switches (Cisco 2960)

1 Layer 3 switch

2 PCs with terminal emulation program Putty

Console cable to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Configure Device Basic Settings

Configure R1.

Configuration tasks for R1 include the following:

| Configuration Item or Task | Specification | Points |
| --- | --- | --- |
| Disable DNS lookup |  |  |
| Router name | <your initials>-R1 e.g MH-R1 |  |
| Encrypted privileged EXEC password | class |  |
| Console access password | cisco |  |
| Encrypt the plain text passwords. |  |  |
| MOTD banner | Unauthorized Access is Prohibited! |  |
| Interface G0/0/1 | Set the description Set the Layer 3 IPv4 address. Refer to the Addressing Table for IPv4 address information. Activate Interface |  |
| Configure 802.1Q subinterface .12 on G0/0/0 | Description Management. Assign VLAN 12. Refer to the Addressing Table for IPv4 address information. |  |
| Configure 802.1Q subinterface .31 on G0/0/0 | Description Sales. Assign VLAN 31. Refer to the Addressing Table for IPv4 address information. |  |
| Configure 802.1Q subinterface .32 on G0/0/0 | Description Marketing. Assign VLAN 32. Refer to the Addressing Table for IPv4 address information. |  |
| Activate Interface G0/0/0 |  |  |
| Interface Loopback 1 (LAN) | Set the Layer 3 IPv4 address. Refer to the Addressing Table for IPv4 address information. |  |
| Interface Loopback 2 (LAN) | Set the Layer 3 IPv4 address. Refer to the Addressing Table for IPv4 address information. |  |
| Interface Loopback 3 (LAN) | Set the Layer 3 IPv4 address. Refer to the Addressing Table for IPv4 address information. |  |

Configure S3.

Configuration tasks for S3 include the following:

| Configuration Item or Task | Specification | Points |
| --- | --- | --- |
| Disable DNS lookup |  |  |
| Router name | <your initials>-S3 e.g MH-S3 |  |
| Interface G0/1 | Set the description Set the Layer 3 IPv4 address. Refer to the Addressing Table for IPv4 address information. Activate Interface |  |
| Interface G0/2 | Set the description Set the Layer 3 IPv4 address. Refer to the Addressing Table for IPv4 address information. Activate Interface |  |
| Interface Loopback 0 (Simulated Internet connection) | Set the description. Configure the IP address as per the addressing table. |  |
| Default route | Configure a static default route out Lo0. |  |

Configure R3.

Configuration tasks for R3 include the following:

| Configuration Item or Task | Specification | Points |  |
| --- | --- | --- | --- |
| Disable DNS lookup |  |  |  |
| Router name | <your initials>-R3 e.g MH-R3 |  |  |
| Interface G0/1 | Set the description Set the Layer 3 IPv4 address. Refer to the Addressing Table for IPv4 address information. Activate Interface |  |  |
| Configure 802.1Q subinterface .12 on G0/0 | Description Management. Assign VLAN 12. Refer to the Addressing Table for IPv4 address information. |  |  |
| Configure 802.1Q subinterface .31 on G0/0 | Description Sales. Assign VLAN 31. Refer to the Addressing Table for IPv4 address information. |  |  |
| Configure 802.1Q subinterface .32 on G0/0 | Description Marketing. Assign VLAN 32. Refer to the Addressing Table for IPv4 address information. |  |  |
| Activate Interface G0/0 |  |  |  |
| Interface Loopback 4 (LAN) | Set the Layer 3 IPv4 address. Refer to the Addressing Table for IPv4 address information. |  |  |
| Interface Loopback 5 (LAN) | Set the Layer 3 IPv4 address. Refer to the Addressing Table for IPv4 address information. |  |  |

Configure S1.

Configuration tasks for S1 include the following:

| Configuration Item or Task | Specification | Points |
| --- | --- | --- |
| Switch name | <your initials>-S1 e.g MH-S1 |  |
| Create the VLAN database | Use Topology VLAN Key table to create and name each of the listed VLANS. |  |
| Assign an IPv4 address to the management SVI. | Refer to the Addressing Table for IPv4 address information. |  |
| Assign the default-gateway. | Refer to the Addressing Table. |  |
| Force trunking on the interface connected to R1. | Use VLAN 1 as the native VLAN. |  |
| Disable the Dynamic Trunking Protocol (DTP) on all ports except F0/1, F02, F0/5 | Make sure ports are configured as access ports. |  |
| Assign VLAN 31 to the port connected to PC-A. |  |  |
| Configure port security on all non-trunk ports | Permit 3 MAC addresses, Sticky, Violation mode = shutdown |  |
| Shutdown all unused ports as per the diagram. |  |  |

Configure S2

Configuration tasks for S2 include the following:

| Configuration Item or Task | Specification | Points |
| --- | --- | --- |
| Switch name | <your initials>-S2 e.g MH-S2 |  |
| Create the VLAN database | Use Topology VLAN Key table to create and name each of the listed VLANS. |  |
| Assign an IPv4 address to the management SVI. | Refer to the Addressing Table for IPv4 address information. |  |
| Assign the default-gateway. | Refer to the Addressing Table. |  |
| Force trunking on the interface connected to R3. |  |  |
| Disable the Dynamic Trunking Protocol (DTP) on all ports except F0/1, F0/2, F0/5 | Make sure ports are configured as access ports. |  |
| Assign VLAN 32 to the port connected to PC-B |  |  |
| Configure port security on all non-trunk ports | Permit 3 MAC addresses, Sticky, Violation mode = shutdown |  |
| Shutdown all unused ports. |  |  |

Configure LAN Redundancy and Link Aggregation

Configure Spanning Tree on S1.

Configuration tasks for S1 include the following:

| Configuration Item or Task | Specification | Points |
| --- | --- | --- |
| Configure Rapid PVST+. |  |  |
| Configure as secondary root bridge for the Sales VLAN |  |  |
| Configure as primary root bridge for the Marketing VLAN |  |  |
| Configure as primary root bridge for the Management |  |  |

Configure Spanning Tree on S2.

Configuration tasks for S2 include the following:

| Configuration Item or Task | Specification | Points |
| --- | --- | --- |
| Configure Rapid PVST+. |  |  |
| Configure as primary root bridge for the Sales VLAN |  |  |
| Configure as secondary root bridge for the Marketing VLAN |  |  |
| Configure as secondary root bridge for the Management |  |  |

Configure EtherChannel between S1 and S2.

Configuration tasks include the following:

| Configuration Item or Task | Specification | Points |
| --- | --- | --- |
| On S1, configure a PAGP EtherChannel on interfaces connected to S2. | Use group 3 and enable ensure PAGP actively tries to negotiate an etherchannel. |  |
| On S1, configure port channel 3 as an 802.1q Trunk |  |  |
| On S2, configure a PAGP EtherChannel on interfaces connected to S1. | Use group 3 and enable PAGP only if a PAGP device is detected. |  |
| On S2, configure port channel 3 as an 802.1q Trunk |  |  |

Configure HSRP

Configure HSRP on R1

Configuration tasks include the following:

| Configuration Item or Task | Specification | Points |
| --- | --- | --- |
| Configure standby group 12 for VLAN 12 | VIP address of 172.10.12.254 Preempt Priority 110 |  |
| Configure standby group 31 for VLAN 31 | VIP address of 172.10.31.254 Preempt Priority 150 |  |
| Configure standby group 32 for VLAN 32 | VIP address of 172.10.32.254 Preempt Priority 150 |  |

Configure HSRP on R3

Configuration tasks include the following:

| Configuration Item or Task | Specification | Points |
| --- | --- | --- |
| Configure standby group 12 for VLAN 12 | VIP address of 172.10.12.254 Preempt Priority 150 |  |
| Configure standby group 31 for VLAN 31 | VIP address of 172.10.31.254 Preempt Priority 110 |  |
| Configure standby group 32 for VLAN 32 | VIP address of 172.10.32.254 Preempt Priority 110 |  |

DHCP

Configure R1 as the DHCP server for VLANs 31 and 32.

Configuration tasks for R1 include the following:

| Configuration Item or Task | Specification | Points |
| --- | --- | --- |
| Reserve the first 20 IP addresses in VLAN 31 for static configurations |  |  |
| Reserve the first 20 IP addresses in VLAN 32 for static configurations |  |  |
| Create a DHCP pool for VLAN 31 | Name: ACCT DNS-Server: 10.10.10.11 Domain-Name: BIT-sba.com Set the default gateway. |  |
| Create a DHCP pool for VLAN 32 | Name: ENGNR DNS-Server: 10.10.10.11 Domain-Name: BIT-sba.com Set the default gateway. |  |
| Verify PC-A & PC-B have received an IP |  |  |

Configure OSPF Dynamic Routing Protocol

Total points:

Configure OSPFv2 on R1.

Configuration tasks for R1 include the following:

| Configuration Item or Task | Specification | Points |
| --- | --- | --- |
| OSPF Process ID. | 1 |  |
| Router ID. | 1.1.1.1 |  |
| Change the default cost reference bandwidth to support Gigabit interface calculations. |  |  |
| Advertise directly connected networks and establish an adjacency with S3. | Use classless network addresses. Assign G0/0/1 to Area 0 Assign G0/0/0 sub interfaces to Area 0. Assign Loopback interfaces to Area 1. |  |
| Set all loopback interfaces as passive. |  |  |
| Configure an inter-area summary route for the networks in Area 1. |  |  |
| Adjust the OSPF metric cost of G0/0/1. | Cost: 15000 |  |
| Create an OSPF MD5 key on G0/0/1. | Key: 1 Password: CISCO |  |
| Apply MD5 authentication to G0/0/1. |  |  |

Configure OSPFv2 on S3.

Configuration tasks for S3 include the following:

| Configuration Item or Task | Specification | Points |
| --- | --- | --- |
| OSPF Process ID. | 1 |  |
| Router ID. | 2.2.2.2 |  |
| Change the default cost reference bandwidth to allow for Gigabit interfaces. |  |  |
| Configure S3 to establish an OSPF adjacency with R1 & R3 | Use classless network addresses. Assign G0/1 to Area 0 Assign G0/2 to Area 0 |  |
| Propagate the default route via OSPF |  |  |
| Adjust the OSPF metric cost of G0/1. | Cost: 15000 |  |
| Create an OSPF MD5 key on G0/1. | Key: 1 Password: CISCO |  |
| Apply MD5 authentication on G0/1. |  |  |

Configure OSPFv2 on R3.

Configuration tasks for R3 include the following:

| Configuration Item or Task | Specification | Points |
| --- | --- | --- |
| OSPF Process ID. | 1 |  |
| Router ID. | 3.3.3.3 |  |
| Change the default cost reference bandwidth to support Gigabit interface calculations. |  |  |
| Configure R3 to establish an OSPF adjacency with S3 | Use classless network addresses. Assign G0/1 to Area 0 |  |
| Advertise directly connected networks. | Use classless network addresses Assign G0/0 sub interfaces to Area 0 |  |
| Configure an inter-area summary route for the networks in Area 2. |  |  |

Verify Network Connectivity

Use the listed command to verify that network is working as expected.

Verify end-to-end connectivity.

Take corrective action if results are other than expected.

| From | Command | To | Expected Results | Points |
| --- | --- | --- | --- | --- |
| PC-A | ping | PC-B | Ping should be successful. |  |
| PC-A | ping | 200.7.12.18 | Ping should be successful. |  |
| PC-B | ping | 200.7.12.18 | Ping should be successful. |  |

Confirmation

Take corrective action if results are other than expected.

| From | Command | Expected Results | Points |
| --- | --- | --- | --- |
| R2 | show ip ospf neighbors | Confirm neighbor relationship with R1 & R3 |  |
| R1 | show standby brief | Confirm correct active router for each group |  |

Note: It may be necessary to disable the PC firewall for pings to be successful.

Instructor Sign-off Part 4: ______________________

Cleanup

NOTE: DO NOT PROCEED WITH CLEANUP UNTIL YOUR INSTRUCTOR HAS GRADED YOUR SKILLS EXAM AND HAS INFORMED YOU THAT YOU MAY BEGIN CLEANUP.

When practicing, ensure to erase configuration and tidy cables for the next user

Before turning off power to the routers, remove the NVRAM configuration files (if saved) from all devices.

Routers & Switches:

erase startup-config

reload