## Config

##### Network Summary (all IP subnets)

| Network Name | VLAN | Number of host addresses required | Network Address | Subnet Mask | Max Number of Hosts Possible | Assignable Range | Gateway Address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sales | 10 | 23 | 10.8.0.0/27 | 255.255.255.224 | 30 | 10.8.0.1 - 10.8.0.30 | 10.8.0.3 |
| Admin | 20 | 23 | 10.8.0.32/27 | 255.255.255.224 | 30 | 10.8.0.33 - 10.8.0.62 | 10.8.0.35 |
| Server | 30 | 13 | 10.8.0.64/28 | 255.255.255.240 | 14 | 10.8.0.65 - 10.8.0.78 | 10.8.0.67 |
| Management | 99 | 14 | 10.8.0.80/28 | 255.255.255.240 | 14 | 10.8.0.81 - 10.8.0.94 | 10.8.0.83 |
| IPv6Sales | 10 | 23 | 2001:DB8:100:1::/64 | /64 | 1.84x10^19 |  |  |
| IPv6Admin | 20 | 23 | 2001:DB8:100:2::/64 | /64 | 1.84x10^19 |  |  |

##### HQ Router1 Name:  HO_Router1

| Interface/Sub Interface | Network Name | Description and Purpose | Network Address | Interface IP Address | Subnet Mask |
| --- | --- | --- | --- | --- | --- |
| G0/0.10 | Sales | Gateway for vlan 10 | 10.8.0.0 | 10.8.0.1 | 255.255.255.224 |
| G0/0.20 | Admin | Gateway for vlan 20 | 10.8.0.32 | 10.8.0.33 | 255.255.255.224 |
| G0/0.30 | Sever | Gateway for vlan 30 | 10.8.0.64 | 10.8.0.65 | 255.255.255.240 |
| G0/0.99 | Management | Gateway for vlan 99 | 10.8.0.80 | 10.8.0.81 | 255.255.255.240 |
| G0/1 | WAN | WAN link to site 2 | 172.16.1.0/30 | 172.16.1.1 | 255.255.255.252 |
| Se0/0/0 | internet | Serial link to ISP | 192.168.1.0/30 | 192.168.1.2 | 255.255.255.252 |
| Se0/0/0 | internet | Serial link to ISP | 2001:DB8:100:FE:: | 2001:DB8:100:FE::1 | /64 |

##### HQ Router2 Name: HO_Router2

| Interface/Sub Interface | Network Name | Description and Purpose | Network Address | Interface IP Address | Subnet Mask |
| --- | --- | --- | --- | --- | --- |
| G0/0.10 | Sales | Gateway for vlan 10 | 10.8.0.0 | 10.8.0.2 | 255.255.255.224 |
| G0/0.20 | Admin | Gateway for vlan 20 | 10.8.0.32 | 10.8.0.34 | 255.255.255.224 |
| G0/0.30 | Sever | Gateway for vlan 30 | 10.8.0.64 | 10.8.0.66 | 255.255.255.240 |
| G0/0.99 | Management | Gateway for vlan 99 | 10.8.0.80 | 10.8.0.82 | 255.255.255.240 |
| G0/1 | WAN | WAN link to site 2 | 172.16.1.4/30 | 172.16.1.5 | 255.255.255.252 |
| Se0/0/0 | internet | Serial link to ISP | 192.168.4.0/30 | 192.168.1.6 | 255.255.255.252 |

##### HQ Distribution Switch1 Name: DisSwitch1

| Switch Mgmt IP address: 10.8.0.84 | Management VLAN: vlan99 |
| --- | --- |

| Port/Number | Description and Purpose | Trunk / Access | VLAN / VLANs allowed |
| --- | --- | --- | --- |
| Fa0/1-2 | AccessSwitch1 | Trunk | 10,20,30,99 |
| Fa0/3-4 | AccessSwitch2 | Trunk | 10,20,30,99 |
| Fa0/5-6 | Distribution Switch2 | Trunk | 10,20,30,99 |
| Gig0/1 | Router 1 | Trunk | 10,20,30,99 |

##### HQ Distribution Switch2 Name: DisSwitch2

| Switch Mgmt IP address: 10.8.0.85 | Management VLAN: vlan99 |
| --- | --- |

| Port/Number | Description and Purpose | Trunk / Access | VLAN / VLANs allowed |
| --- | --- | --- | --- |
| Fa0/1-2 | AccessSwitch2 | Trunk | 10,20,30,99 |
| Fa0/3-4 | AccessSwitch1 | Trunk | 10,20,30,99 |
| Fa0/5-6 | Distribution Switch1 | Trunk | 10,20,30,99 |
| Gig0/1 | Router 2 | Trunk | 10,20,30,99 |

##### HQ AccessSwitch1 Name: AccSwitch1

| Switch Mgmt IP address: 10.8.0.86 | Management VLAN: vlan99 |
| --- | --- |

| Port/Number | Description and Purpose | Trunk / Access | VLAN / VLANs allowed |
| --- | --- | --- | --- |
| Fa0/1-2 | Distribution Switch1 | Trunk | 10,20,30,99 |
| Fa0/3-4 | Distribution Switch2 | Trunk | 10,20,30,99 |
| Fa0/10 | PC 0 Sales | Access | 10 |
| Fa0/11 | PC1 Admin | Access | 20 |
| Fa0/12 | Server 1 | Access | 30 |

##### HQ AccessSwitch2 Name: AccSwitch2

| Switch Mgmt IP address: 10.8.0.87 | Management VLAN: vlan99 |
| --- | --- |

| Port/Number | Description and Purpose | Trunk / Access | VLAN / VLANs allowed |
| --- | --- | --- | --- |
| Fa0/1-2 | Distribution Switch2 | Trunk | 10,20,30,99 |
| Fa0/3-4 | Distribution Switch1 | Trunk | 10,20,30,99 |
| Fa0/10 | PC 2 Sales | Access | 10 |
| Fa0/11 | PC3 Admin | Access | 20 |
| Fa0/12 | Server 2 | Access | 30 |

##### Site2 Switch Name: Site2L3

| Interface | Network Name | Description and Purpose | Network Address | Interface IP Address | Subnet Mask |
| --- | --- | --- | --- | --- | --- |
| Vlan 10 | Admin | Admin vlan default gateway | 10.9.0.0 | 10.9.0.1 | 255.255.255.240 |
| Vlan20 | Sales | Sales vlan default gateway | 10.9.0.16 | 10.9.0.17 | 255.255.255.240 |
| G0/1 | WAN | WAN link to HO Router 1 | 172.16.1.0/30 | 172.16.1.2 | 255.255.255.252 |
| G0/2 | WAN | WAN link to HO Router 2 | 172.16.1.4/30 | 172.16.1.6 | 255.255.255.252 |

##### ISP Router Name: ISP

| Interface | Network Name | Description and Purpose | Network Address | Interface IP Address | Subnet Mask |  |
| --- | --- | --- | --- | --- | --- | --- |
| Se0/0/0 | internet | Serial link to H0 router 1 | 192.168.1.0/30 | 192.168.1.1 | 255.255.255.252 |  |
| Se0/0/1 | internet | Serial link to H0 router 2 | 192.168.1.4/30 | 192.168.1.5 | 255.255.255.252 |  |
| Se0/0/0 | internet | Serial link to H0 router 1 | 2001:DB8:100:FE::/64 | 2001:DB8:100:FE::2/64 | /64 |  |
| G0/0 |  | Link to the isp PC | 209.100.100.0 | 209.100.100.3 |  | 255.255.255.248 |
| G0/0 |  | Link to the isp PC IPV6 | 2001:DB8:100:AA:: | 2001:DB8:100:AA:1/64 |  |  |

Static Server Addresses

| Server Name | IP address | Subnet Mask | Gateway |
| --- | --- | --- | --- |

## Questions

S2-4. Describe how the root bridge, designated ports and root ports would be determined by default. Provide captured output to confirm the root bridge in your topology.

By default the root id is determined by the lowest mac address if all the portieres are the same. The Root Port On each non‑root switch is the port with lowest path cost to the Root Bridge. The Designated Port on each segment is the port on the switch with the lowest path cost to the Root Bridge.

Distribution switch 1

Distribution switch 2

S2-7. Ensure all appropriate ports have port-fast configured. What is the purpose of this command?

Port-fast skips the listening and learning steps of STP which give a delay and goes straight to forwarding this allows PCs get instant network connectivity

S2-8. Ensure all appropriate ports have BPDU-guard configured. What is the purpose of this command?

BPDU Guard is used on PortFast-enabled access ports to protect the network from accidental or malicious STP changes.

S2-10. In your documentation provide captured output showing which ports have been aggregated and the type of aggregation (for all port channels).

S3-4. Ensure pre-emption is supported. Document what this does.

HSRP pre‑emption allows a router with the higher priority to take back the Active role if it comes online after a failure or reboot.

S3-5. Show capture output that displays the standby status on both HO routers.

S3-6. Show output evidence confirming the OSPF adjacencies are operational between the two HO routers

S3-7. Research the function of DR & BDR roles within OSPF. Explain why they exist and the default process for DR election.

OSPF uses a Designated Router and Backup Designated Router on multi‑access networks to reduce the number of adjacencies and minimise LSA flooding. Instead of every router forming adjacencies with every other router, all routers form adjacencies only with the DR and BDR. The DR distributes LSAs for the segment, while the BDR takes over immediately if the DR fails. The DR/BDR election is based first on the highest OSPF priority, and if tied, the highest router ID. The DR is elected first, then the BDR, and the election is non‑pre-emptive meaning a new router with a higher priority will not replace the existing DR unless the DR goes down.

S5-2. Show output evidence confirming the OSPF adjacencies are operational

Reefer to QS3-6 images

S5-4. Configure the Site 2 L3 Switch to advertise a single OSPF summary address representing the Site2 user VLAN networks. Provide evidence of the summary routes.

S5-5. Configure OSPF such that the Head Office Router 2 is the preferred path to and from Site2. Explain how you have done this, do not rely on default behaviour.

I have don this by increasing the cost of the router to go to router 1  and decreasing the cost to go to router 2

S6-3. Document output from the HO Router2 confirming DHCP leases for HO and Site2.

S6-4. Explain the difference between stateless and stateful DHCPv6.

Stateless DHCPv6 allows hosts to generate their own IPv6 address using SLAAC, Stateful DHCPv6 works like IPv4 DHCP, where the DHCPv6 server assigns the full IPv6 address and configuration details

S8-3. Provide evidence of IPv6 connectivity between a PC in the HO Admin VLAN and the HO Sales VLAN.

S8-4. Provide evidence of IPv6 connectivity between HO user PCs and the Internet Test PC.

## Testing / Final Connectivity

| PING TABLE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | HO Admin PC | HO Sales PC | HO Server | Site2 Admin PC | Site2 Sales PC | Internet PC |
| HO Admin PC | N/A | pass | pass | Pass | pass | PASS |
| HO Sales PC | pass | N/A | pass | Pass | pass | PASS |
| HO Server | pass | pass | N/A | pass | pass | pass |
| Site2 Admin PC | pass | pass | pass | N/A | pass | PASS |
| Site2 Sales PC | pass | pass | pass | pass | N/A | PASS |
| Internet PC | pass | pass | pass | pass | pass | N/A |